Fulton County Transit Authority

Low Floor Minivan Invitation for Bid (IFB)

The Fulton County Transit Authority (FCTA) is accepting bids for one (1) – twelve (12) new model year 2025 or newer low floor ramp equipped minivans (see specifications below) at its administrative facility located at 302 Eastwood Drive, Fulton, Kentucky. Firms should be willing to sign the Federal Model Clauses, Vendor Agreement, and be registered in https://sam.gov/SAM/.  All bids are subject to a financial assistance agreement between FCTA, the Kentucky Transportation Cabinet’s Office of Transportation Delivery, and the Federal Transit Administration.

All bids may be turned into the Office of the Executive Director, Fulton County Transit Authority, 302 Eastwood Drive, Fulton. KY. 42041 or may be submitted to k.kelley@fultontransit.com, no later than 9:00 am CST on January 21, 2026.  Bids cannot be accepted in any other manner.  Bids will be opened thereupon or about 9:00 am CST, January 21, 2026, in the office of the Executive Director, 302 Eastwood Drive, Fulton, KY 42041. Bids must be received by the designated date and time, and none will be considered thereafter.

The Fulton County Transit Authority reserves the right to reject any and all bids received, and to select that bid which it determines to be in its best interest.  The award of this contract will be made on the basis of the lowest evaluated price as permitted in KRS 45A.365.  Questions should be directed to Kevin Kelley at 270-472-0662.

Fulton County Transit Authority Low Floor Ramp Equipped Minivan Specifications

Minimum Requirements-must meet or exceed listed specifications; equivalents must be noted.

FCTA Low Floor Ramp Equipped Minivan Specifications

NOTE: Accepting options to purchase new Lowered Floor Mini-Van with Side Entrance In floor Ramp for Wheelchair Access with seating arrangements for 3 Ambulatory and 2 Wheelchair Positions.

# DESCRIPTION:

These specifications describe a new Lowered Floor Mini-Van with Side Entrance In Floor Ramp for Wheelchair Access, which will be used to transport passengers in both rural and urban areas. The Low floor minivan shall be of substantial and durable construction in all respects, with particular attention given to features, which will provide the safest possible lowered floor minivan for transporting passengers. Prior to entering into a formal contract, the bidder must provide certification of compliance with the Buy America requirements of the Surface Transportation Assistance Act of 1982, as amended in the regulations in 49 CFR Parts 661 and 663; a self-certification for meeting requirements of the Federal Motor Vehicle Safety Standards (FMVSS); and state that it is a responsible manufacturer with the capability to produce a vehicle that meets the purchaser’s requirements.

ISO 9001:2015: Van converter must be ISO 9001:2015 certified for the design, manufacture and assembly of wheelchair accessible vehicles. ISO (International Organization for Standardization) is an international program that ensures quality and safety.

Altoona Bus Testing Report: The converted minivan must have been submitted to the Altoona Bus Test Center for a 4 yr./ 100,000-mile Surface Transportation and Uniform Relocation Assistance Act (STURAA) test. Testing must have been completed on current body style being converted. A copy of the test report shall be made available with bid submission.

CARB Compliance: Vehicle as converted must be California Air Resources Board (CARB) compliant. The CARB Executive Order number must be printed on the identification label and affixed to the fuel tank.

FCA (Chrysler Quality Assurance Certification: Converter must be certified under the FCA’s Ram Commercial Q Pro program.

The pre-award and post-delivery audits for compliance with the requirements of Buy America will be conducted by a person who is not an employee or agent of the manufacturer.

The bidder shall certify that all equipment provided meets or exceeds all state and federal requirements applicable to the equipment. For example, equipment shall meet USDOT, ICC, FMVSS, Buy American Requirements, OSHA and Kentucky Motor Vehicle Requirements as to highway safety and other applicable regulations. Required model clauses are attached to bid package.

Quantity: minimum of one (1) vehicle to be purchased with twelve (12) options to purchase additional vehicles for a maximum of twelve (12) vehicles per specification. In addition, Fulton County Transit Authority (FCTA) has the right to assign a portion of the contract awarded, to allow other transit agencies to purchase any remaining options.

Failure of FCTA to specifically identify Federal and State regulations in its specification does not relieve the bidder of the responsibility to meet them.

# DETAILED SPECIFICATIONS:

Materials

All materials used in manufacturing of the Lowered Floor Mini-Van shall conform in all respects to American Society of Testing Materials, Society of Automotive Engineers or similar association standards. Materials used shall be of first quality and shall be exactly duplicated in manufacture, design and construction on each Low Floor Mini-Van. All units or parts not specified shall be manufacturer’s best quality and shall conform in materials, design, or workmanship to the best practice known in the automotive industry. All parts shall be new and in no case will used, reconditioned or obsolete parts be accepted. The parts on all vehicles provided by the same manufacturer should be interchangeable.

# Capacity

Lowered Floor Mini-Van with Side Entrance In Floor Ramp for Wheelchair Access must be able to accommodate the following load:

1.  3 Ambulatory and 2 Wheelchairs including driver.

DIMENSIONS:

| Description | Requirements |
| --- | --- |
| Mini-Van | Lowered Floor Side In Floor Ramp Equipped |
| Body Warranty | 3 years/36,000 Miles Structural Warranty |
| Overall Exterior Length | 203.8” Maximum |
| Overall Exterior Width | 79.6” Maximum |
| Overall Height | 81” Maximum |

CHASSIS EQUIPMENT:

| Description | Requirements |
| --- | --- |
| Model Year | 2025 or Newer |
| Wheelbase | 121.6” Maximum |

| G.V.W.R | 6,055 lbs. Minimum |
| --- | --- |
| Engine | 3.6L V-6 Gasoline Engine Minimum |
| Transmission | Nine (9) Speed Automatic w/ Overdrive Minimum |
| Steering | Power |
| Suspension | Fully Independent front suspension with struts and heavy-duty rear springs. |
| Brakes | 4- Wheel Anti-Lock Disk Brakes |
| Batteries | Dual OEM Batteries; Battery 1 650 CCA MINIMUM; Battery 2 200 CCA Minimum |
| Alternator | OEM 180 Amps Minimum |
| Bumper Front/Rear & Grill | Manufacturer’s standard front and rear bumpers with matching bumper guards |
| Wheels and Tires | Tires and Wheels shall be OEM and compatible with each vehicle and each GVWR. Tire shall be steel belted radials, all season type. Tire size shall be P235/65HR17 or equivalent |
| Rear Axle Capacity | 2,400 lbs. Minimum |
| Front Axle Capacity | 2,400 lbs. Minimum |

| Description | Requirements |
| --- | --- |
| Spare Tire | OEM Inflatable compact spare tire shall be supplied mounted in the OEM location in the vehicle sidewall. Tire changing tools shall be supplied |
| Fuel Capacity | 19 Gallons Minimum |
| Cooling System | Heavy Duty radiator with coolant recovery system, engine oil cooler and transmission cooler |
| Heating | Standard factory front and rear heat w/ rear window defroster |
| Insulation | Completely insulated extended length |
| Exterior Lighting | Exterior lighting normally provided by the manufacturer as standard equipment. Lighting over wheelchair ramp door shall be in accordance with ADA requirements |
| Interior Lighting | Front, Intermediate and rear courtesy/reading lamps shall be provided |

| Air Conditioning | Standard factory front and rear air conditioning w/ manual Dual Zone Temperature Control for front and rear |
| --- | --- |
| Key Fob | Keyless entry; remote start |
| Power Package | Power Windows, Doors, Locks, w/ Keyless Entry |
| Instrument Panel | Speedometer, odometer, fuel gauge, voltage gauge, oil pressure gauge, tachometer and engine temperature indicator |
| Fresh Air Intake | Manufacturer’s standard |
| Wipers | Variable Control System. Windshield wiper on lift gate shall be provided. |
| Air Bag | Driver and passenger sides |
| Steering Features | Tilt Wheel w/Cruise Control |
| Radio | OEM U-connect 5 AM/FM Radio With Clock In OEM Chassis Location In Dash |
| Rear Door | Hinged rear door that can be opened from inside and out shall be provided |
| Passenger Doors | Side doors shall be sliding. Height of wheelchair/ramp entrance opening shall be a minimum of 56” and the width shall accommodate a 30” platform ramp |
| Glass | In all doors and panels; windshield may have normal factory tint; driver’s window and all passenger windows shall have 33% tint with privacy glass in passenger compartment |

| Description | Requirements |
| --- | --- |
| Seating | Driver’s seat to be standard OEM with armrest. Front passenger seat is to be removable to allow for forward facing wheelchair position, high backed with armrest. Additional passengers include an aftermarket 3- person stationary bench seat across the rear of the van. Two forward facing wheelchair positions. Wheelchair positions must meet ADA requirements. Driver and Co-pilot seats to be upholstered in aftermarket heavy-duty transit grade black vinyl, (OEM black cloth or OEM black leatherette vinyl not acceptable). 3rd row bench seats upholstered in black vinyl material that meet FMVSS 302 burn resistance requirements. |

| Interior | All interior panels, upholstery, and trim shall harmonize with exterior vehicle, black standard |
| --- | --- |
| Assist Handle | If offered by manufacturer |
| Retractable Seat and Shoulder Belts | All ambulatory seating positions; must meet all applicable FMVSS requirements and be able to accommodate child safety seats. Extra-long seat belts with shoulder harness and lap belts shall be provided at wheelchair positions to accommodate wheelchair and passenger. All vehicles must have a pouch mounted to store these devices when not in use. Two sets of ambulatory seatbelt extensions shall be provided |
| Paint | White Paint for exterior. Flares shall be constructed of formed black TPO plastic. A molded step shall be incorporated into the driver and passenger flares to aid entry. Beneath the flares molded step surface shall be an adjustable steel support structure capable of supporting 400 lbs. with less than 1/8” deflection, which fastens directly to the vehicles body structure. The flares molded step surface shall have a minimum clear vertical width of 4.5”, a minimum length of 18” and utilize an anti-skid material which defines the step surface. No dealer decals or emblems on vehicle. |

| Description | Requirements |
| --- | --- |
| Mirrors | Right- and left-hand powered side mirrors. Adequate rearview mirror shall be installed for drivers of interior and rear of vehicle. |
| Undercoating | Complete undercoating with corrosion protection with minimum 5-year warranty. |
| Safety Equipment | 10” In-dash Back-up camera to be provided. Back up sonar and back up alarm must be installed in each vehicle. Mounted 16-unit First Aid Kit , mounted 5 lb. fire extinguisher, reflective triangle kit containing 3 triangles, seatbelt cutter, blood spill clean-up kit properly placed and secured. |

CONSTRUCTION:

The entire body structure shall be built as an integral unit adequately reinforced at all joints and corners where stress concentration may occur to adequately carry required loads and withstand road shock. Steel safety frame (floor, walls, roof, front, and rear) shall be securely welded together to provide an integral one-piece body structure. Exterior body skin to be OEM high strength steel dipped in galvanic coating. No fasteners to be exposed to exterior finish.

# MODIFICATIONS:

- All modifications shall be affected in good workmanlike manner. Sheet metal fit-ups shall be properly executed and concealed. Paint finish and window glass shall be free of any defects due to welding or re-assembly.

- Modifications to OEM floor plan/frame shall be of highest possible quality construction. Modification of floor and frame shall provide reinforcement to the degree that the structural integrity of the OEM vehicle is not degraded. Any such modifications shall be properly sealed to prevent entrance of exhaust fumes, moisture, and dust.

- All welds shall be made in a workmanlike manner, properly fused, of ample penetration, and shall, on all exposed surfaces where practicable, be smoothly finished. Intermittent or spot welds shall be spaced and proportioned as to provide ample strength for the purpose. All welded joints shall be spaced and proportioned to provide ample strength for the purpose. All welded joints shall be cleaned and primed. All welds shall be relatively free of slag inclusions, undercut, roll, blowholes, unfilled craters, improper fit-up and porosity. Fillet weld sizes shall be at least equal to the thickness of the least of the joined plates.

# LOWERED FLOOR MATERIAL SPECIFICATIONS:

- All flooring modifications shall be executed in 14 and 16-gauge aluminized steel. Such modifications shall not degrade the structural integrity of the OEM vehicle.

- The lowered floor shall be securely fastened to steel chassis or body and covered from wall to wall with industrial grade, non-slip vinyl flooring. The sub-floor shall be thermoplastic panels, consisting of a polypropylene honeycomb core with chopped glass reinforced facing on both sides - providing a durable water-resistant base with superior

strength to weight ratio. The panels provide a smooth surface for flooring attachment and minimize interior noise.

- Due to lowering of the floor, the fuel tank shall be relocated behind rear axle. A fuel tank mounting structure shall be provided that has been fully crash tested and documented to prove fuel system integrity (FMVSS 301). A Heat shield must be installed between the gas tank and exhaust on each vehicle.

- Floor covering shall be slip resistant vinyl flooring, constructed with aluminum oxide, silicon carbide, quartz and multiple colored PVC chip blended throughout a high-quality vinyl wear surface for better depth perception for sight impaired (top coating is not acceptable). Bacteriostats will be incorporated providing all exposed surfaces with excellent anti-bacterial properties. Minimum thickness of 2.2 millimeters (combination of flooring and backing material will not be accepted). Altro trans floor or approved equal. Manufacturer determines if the flooring is equal.

# FUEL TANK:

Tank, fuel lines and hardware must meet all current FMVSS, including FMVSS 301, as well as all EPA requirements and must be OEM equivalent in connection types, etc. The use of worm clamps is limited to that of the OEM. Tank shall be calibrated with the OEM dash fuel gauge.

# ELECTRICAL SYSTEM:

- Electrical system and equipment shall comply with all applicable FMVSS and shall also conform to all the applicable SAE recommended standards and practices.

- Electrical system shall incorporate a warning light and audible buzzer, to indicate a door not fully closed.

- All wiring shall be secured to the body or frame with straps in order to prevent sagging and movement, which results in chafing, pinching, snagging, or any other damage.

# RAMP:

- The ramp shall be a manually operated in floor pull out type. The In-floor ramp allows ambulatory passengers to load and unload without the need to deploy or swing out the ramp. The ramp shall provide a straight inclined surface from the ground to the van floor level. This should be approximately 12.5” from the van floor level to ground level with a ramp slope of 11 degrees (nominal). The ramp shall allow a wheelchair containing a handicapped person to be pushed from the ground level into each van or rolled out of each van down to ground level. Protective edge flanges shall be provided on the ramp to prevent the

wheels of a wheelchair from rolling off the edge of the ramp. No center steps or toe-cleats will be allowed with this conversion.

- The ramp shall have a clear width of at least 30” and length of 63.5” (including the interior ramp cover that acts as parts of the sloping ramp) at minimum, this length is required to achieve the approximate 11-degree slope. The gradient must meet ADA requirements.

- The ramp shall be constructed of ¼” aluminum black powder coated.

- Ramp capacity of 1000 lbs. minimum

- Wheelchair ramp must meet the requirements of ADA.

# RAMP DOOR:

The ramp shall be manual, mounted to the chassis frame without structural changes in the frame and located at the side of the van. The door that the ramp is located shall have a minimum height of 56” and minimum width of 31” (excluding grab handles). Sliding doors will be extended to floor level. Door extensions shall be constructed of aluminum to minimize weight while preserving strength and integrity.

# RAMP MECHANISIM:

Shall be manual type. Nonworking parts, such as platform, frame, and attachment hardware that would not be expected to wear, shall have a safety factor of at least three, based on the ultimate strength of the material.

# RAMP CONTROLS:

- Control interlock. The ramp doors shall be interlocked with the vehicle transmission to ensure the vehicle cannot be shifted out of park when the ramp is deployed or the ramp access door is ajar. This interlock will meet all ADA requirements of 49 CFR 38.2b (2)

- No emergency method, manual or otherwise, shall be capable of being operated in a manner that could be hazardous to the ramp occupant or to the operator

# RAMP PLATFORM SURFACE:

The platform surface shall be free of any protrusions over ¼ inches high and shall be non-skid aluminum alloy or equivalent with a minimum weight capacity of 1000 lbs. The platform shall have a minimum clear width of 30 inches measured from the platform surface to 30 inches above the platform and a minimum clear length of 63.5” inches measured from 2 inches above the surface of the platform.

# SECUREMENT DEVICES:

- Design Load: Securement systems, and their attachments to such vehicles, shall withstand a force in a forward longitudinal direction of 2,500 pounds per securement leg, and a minimum of 5,000 pounds for each mobility aid.

- Location and Size: The securement system shall be placed as near to the accessible entrance as practical and shall have a clear floor area of 30 inches by approximately 44 inches. Not more than 6 inches of the required clear floor space may be accommodated for footrests under another seat provided there is a minimum of 9 inches from the floor to the lowest part of the seat overhanging the space.

- L Track shall be installed to allow for a wheelchair position in either the right front passenger area, when the passenger seat is removed, or the rear passenger area. The rear passenger area shall have two (2) recessed floor mounted L tracks that run from one side of the van to the other.

Securement System: The securement system shall secure common wheelchairs and mobility aids and shall be easily attached by a person familiar with the system and mobility aid and having average dexterity. Wheelchair securing spaces shall be located so that the wheelchairs are locked. Two (2) sets of wheelchair restraints shall be provided. Tie downs for wheelchair positions are to be of the four-point system with four tie downs securing the chair to the floor of the vehicle. 4-point, heavy duty, fully automatic retractable Q-Straint QRT 360 or approved equal (dual knobs) tie-downs built to withstand the higher loads of the WC18 standard and be compatible with WC19 wheelchairs mounted onto L track fitting with Lap & Shoulder Belt Occupant Restraint. This system must be able to withstand a crash test of 30 mph and 20 g’s without damage to the wheelchair or its occupant. Tie down system must be a complete package. Wheelchair Securement System shall have a warranty of five (5) years. A copy of registration of warranty for each vehicle must be provided to the purchasing agency.

- Seat belts and shoulder harnesses with heavy-duty retractors and sleeves to keep belts off the floor shall be provided for each wheelchair position. A detachable shoulder harness that matches the Passenger Occupant Restraint system is required. Standard OEM restraints lap or shoulder system is not acceptable in wheelchair positions.

- Orientation: The securement shall be installed so that the secure wheelchair or mobility aid is facing toward the front of the vehicle.

- Movement: When the wheelchair or mobility aid is secured in accordance with manufacturer’s instructions, the securement system shall limit the movement of an occupied wheelchair or mobility aid to no more than 2 inches in any direction

- Stowage: When not being used, the securement system shall not interfere with passenger movement, shall not present any hazardous condition, and shall be reasonably protected from vandalism.

- Platform Gaps: Any openings between the platform surface and the raised barriers shall not exceed 5/8 inch wide. When the platform is at vehicle floor height with the inner barrier down or retracted, gaps between the forward edge of the ramp shall not exceed 2 inch horizontally and 5/8 inch vertically. The ramp shall be furnished with reflector tape on each side.

- Platform Barriers: The ramp platform shall be equipped with barriers to prevent any of the wheels of a wheelchair or mobility aid from rolling off the platform

- Boarding Direction: The ramp shall permit both inboard and outboard facing of wheelchairs and mobility aids.

- Inside Track: The inside track shall be approximately 54” long and a minimum of 44” wide.

# WARRANTY:

Vehicles shall be in compliance with all state and federal safety laws and is state safety inspected at the time of delivery. The warranty for the basic vehicle shall be the manufacturer’s standard warranty of 3-year/36,000 miles. A Power Train Warranty of 5-year/100,000 miles and Roadside Assistance 5-year/60,000 miles shall be provided. The

warranty for the conversion and modification of the vehicle shall provide that, at a minimum, all repairs and replacements needed due to factory defects shall be furnished and installed promptly without charge by authorized service representatives for three (3) years or 36,000 miles after final delivery of the vehicle, with an option to purchase an extended warranty.

Camera System

The vehicle must have the following camera system installed prior to delivery.

FCTA VEHICLE CAMERA SYSTEM SPECIFICATIONS:

Quantity 	Product			Description

1		DH6H2T0-M		DH6 DVR, 6 HD Channels, Audio, GPS + Antenna, Wi-Fi (AC, AN, 								BGN), 6CH Breakout Harness with 2x3 Video Connectors, Security 								Front Cover with Lock Set, Mounting Plate, Power Harness, 2TB HDD

DVR is to be installed on the side wall of the curb side of the vehicle behind the 3rd row of seats.

Picture 1 on the Camera Installation Instructions.

1		FRGT-DH6-G		Freight, Ground, CE, DH6 System

1		WT2E20S20G0		WT2 Wire Bundle for display of DVR Recording, Video Loss, and								Error. Includes 20 ft. signals harness (WT2) and RGY-Button with

20 ft. harness (HEDM20). Compatible with TH8, DH-Series, and

NH-Series. GPS is not included.

1		HD2D0202AI		Dual Lens Cam, 1080p, 2.1mm lens forward/inward, mic

The Dual Lense Camera is to be mounted to the front windshield to the right of the rear-view mirror.

Picture 2 on the Camera Installation Instructions.

2		HEHD10 			ASSY, CBL, HD, 2X3Micro-Fit M-M, 10ft

1 		HD3U01AN20 		HD Camera, Ultra-Wide, 1.72mm, internal, audio, IR, TDN, 20 ft

harness — Compatible with all TH, DH, & NH16 series recorders,

except TH4C & TH4.

Install the Ultra-Wide camera on the driver’s side of the vehicle to capture loading and unloading of the van and all passengers in the rear of the vehicle.

Picture 3 on the Camera Installation Instructions.

1		ANTK-W22W6 		Dual Band White Wi-Fi Antenna Kit with Two Cables 1ft Length

plus 2x 060-1109 5M (16Ft) WIFI Extension Cables and RP-SMA

Male Terminating Connectors

Install the Wi-Fi Antenna Kit behind the 3rd row of seats in the rear of the vehicle next to the DVR.

Picture 4 on the Camera Installation Instructions.

1 		G-SENSOR-EXT 		G-Sensor, module and cable kit for compatible DVR

# INSPECTIONS:

A representative of Fulton County Transit Authority (FCTA) reserves the right to inspect any vehicle produced by any manufacturer and intended for delivery under this contract. Any deficiencies identified must be rectified prior to acceptance of the vehicle.